DO-178B Temperature Sensor Monitoring Project

# 1. System Requirements Document (SRD)

## Purpose

- Define what the system must do.

- Serve as the source for all software requirements.

- Provide traceable safety and functional requirements for certification.

- Include functional, performance, environmental, and safety aspects.

## Inputs

- Customer requirements (monitor temperature, warn pilot)

- Operational Concept (ConOps)

- Safety assessment (FHA, PSSA, SSA)

- System architecture

- Interface Control Document (ICD)

- Performance and timing specifications

- Environmental requirements

- Regulatory and standard compliance

- Fault handling requirements

## Example Requirements

- SYS-REQ-01: System shall monitor temperature sensor input every 1 second.

- SYS-REQ-02: System shall generate a warning when temperature exceeds 80°C.

- SYS-REQ-03: System shall clear the warning when temperature drops below 75°C.

- SYS-REQ-04: System shall store the over-temperature flag as Boolean.

## Finalization

- Draft creation by system engineers

- Review by software, safety, QA

- Comments resolved

- Baseline under CM, version v1.0 approved

## Traceability

| SRD | HLR |
| --- | --- |
| SYS-REQ-01 | HLR-01 |
| SYS-REQ-02 | HLR-02 |

# 2. High-Level Requirements Document (HLR)

## Purpose

- Define what the software must do to satisfy SRD requirements.

- Translate system requirements into software-specific, testable statements.

- Include functional behavior, performance, and safety requirements.

## Inputs

- SRD, ConOps, ICD

- Safety analysis data

- Performance and timing requirements

- Environmental constraints

## Example HLR Requirements

- HLR-01: Software shall sample temperature sensor every 1 second.

- HLR-02: Software shall detect over-temperature when temp > 80°C.

- HLR-03: Software shall clear over-temperature warning when temp < 75°C.

- HLR-04: Software shall set an over-temperature warning flag as Boolean.

## Finalization

- Peer review by software, verification, QA

- Verify completeness, clarity, and testability

- Comments resolved and baseline under CM

## Traceability

| SRD | HLR |
| --- | --- |
| SYS-REQ-01 | HLR-01 |
| SYS-REQ-02 | HLR-02 |

# 3. Low-Level Requirements Document (LLR)

## Purpose

- Provide detailed, testable logic to implement HLR.

- Specify exact behavior, data structures, and interfaces.

- Serve as basis for code development and verification.

## Inputs

- HLR, safety assessment, timing specifications

- ICD, environmental constraints, performance data

## Example LLR Requirements

- LLR-01: Sample temperature sensor every 1000 ms.

- LLR-02: If temperature > 80°C for 2 consecutive samples, set warning flag TRUE.

- LLR-03: If temperature ≤ 75°C for 2 consecutive samples, clear warning flag FALSE.

- LLR-04: Warning flag stored as Boolean variable.

## Finalization

- Technical review and verification

- Ensure alignment with HLR

- Fix defects, baseline under CM

## Traceability

| HLR | LLR |
| --- | --- |
| HLR-02 | LLR-02 |
| HLR-03 | LLR-03 |

# 4. Software Design Description (SDD) – HLD and LLD

## Purpose

- Provide High-Level Design (HLD) and Low-Level Design (LLD).

- HLD: Define modules, state machines, data flow, and major interfaces.

- LLD: Detail function logic, data structures, algorithms, and sequence diagrams.

- Serve as reference for coding and verification.

## Inputs

- LLR, HLR, coding standards

- Hardware architecture, performance/timing specifications

- Safety and environmental constraints, ICD

## High-Level Design (HLD)

- Module: TempMonitor

- Inputs: Temperature sensor signal

- Outputs: Warning flag

- States: NORMAL, OVER_TEMP

- Data Flow: Sensor → TempMonitor → Warning Flag

- Interfaces: Hardware registers, cockpit display, communication bus

- Description: Major modules, data flow, state transitions, responsibilities

## Low-Level Design (LLD)

- Function: TempMonitor_Update()

- Input: Sensor value

- Logic:

if(sensor_value > 80 && prev_state == NORMAL) count++; 
if(count >= 2) state = OVER_TEMP;
if(sensor_value <= 75 && prev_state == OVER_TEMP) count--; 
if(count <= 0) state = NORMAL;

- Data Structures: state, count, warning_flag

- Interfaces: Sensor register, output flag to cockpit display

- Sequence Diagram: Sensor read → Logic processing → Flag set → Output

## Finalization

- Design review by software, verification, QA

- Ensure all SRD and HLR requirements covered

- Fix comments, baseline under CM, approved v1.0

## Traceability

| SRD | HLR | HLD Module | LLD Function |
| --- | --- | --- | --- |
| SYS-REQ-01 | HLR-01 | TempMonitor | TempMonitor_Update() |
| SYS-REQ-02 | HLR-02 | TempMonitor | TempMonitor_Update() |
| SYS-REQ-03 | HLR-03 | TempMonitor | TempMonitor_Update() |

# 5. Software Verification Cases & Procedures (SVCP)

## Purpose

- Define test cases to verify HLR/LLR and design implementations.

- Include steps, inputs, and expected results.

## Inputs

- HLR, LLR, HLD/LLD, coverage requirements

## Example Test Cases

- TC-01: Temp = 60°C → flag FALSE

- TC-02: Temp = 85°C → flag TRUE

- TC-03: Temp = 70°C → flag FALSE

- TC-04: Temp = 80°C → flag FALSE (boundary)

## Finalization

- Review by verification team + QA

- Ensure all requirements and design covered

- Baseline in CM

## Traceability

| LLR | Test Case |
| --- | --- |
| LLR-02 | TC-02 |
| LLR-03 | TC-03 |

# 6. Software Verification Results (SVR)

## Purpose

- Record test execution results.

- Provide evidence that software satisfies requirements and design.

## Inputs

- SVCP, logs, coverage reports, anomaly reports

## Example Results

| Test Case | Result |
| --- | --- |
| TC-01 | PASS |
| TC-02 | PASS |
| TC-03 | PASS |
| TC-04 | PASS |

Coverage: Statement 100%, Branch 100%

## Finalization

- Review by verification lead + QA

- Fix open anomalies

- Baseline in CM

## Traceability

| Test Case | Result |
| --- | --- |
| TC-02 | PASS |

# 7. Software Validation Document (VDD / SVD)

## Purpose

- Summarize that software meets operational requirements.

- Confirm all requirements tested and passed.

- Provide evidence for certification.

## Inputs

- HLR, LLR, HLD/LLD, SVCP, SVR, traceability, coverage, anomaly reports

## Content

- Validation strategy

- Summary of executed tests

- Coverage statistics

- Confirmation that all requirements validated

- No open anomalies

## Finalization

- Review by verification + QA

- Baseline under CM

- Approved for certification

## Traceability

| SRD | HLR | LLR | HLD | LLD | Code | Test | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SYS-REQ-02 | HLR-02 | LLR-02 | TempMonitor | TempMonitor_Update() | TempMonitor() | TC-02 | PASS |
| SYS-REQ-03 | HLR-03 | LLR-03 | Reset Logic | TempMonitor_Update() | TempMonitor() | TC-03 | PASS |

# Final Traceability Chain

SRD → HLR → HLD → LLD → Code → SVCP → SVR → Software Validation Document