DO-178B Temperature Sensor Monitoring Project

# 1. System Requirements Document (SRD)

## Purpose

- Define what the system must do.

- Serve as the source for all software requirements.

- Provide traceable safety and functional requirements.

## Inputs

- Customer requirements (monitor temperature, warn pilot)

- Operational Concept (ConOps)

- Safety assessment (FHA, PSSA, SSA)

- System architecture

- Interface Control Document (ICD)

- Performance and timing specifications

- Environmental requirements

- Regulatory and standard compliance

- Fault handling requirements

## Example Requirements

- SYS-REQ-01: System shall monitor temperature sensor input every 1 second.

- SYS-REQ-02: System shall generate a warning when temperature exceeds 80°C.

- SYS-REQ-03: System shall clear the warning when temperature drops below 75°C.

- SYS-REQ-04: System shall store the over-temperature flag as Boolean.

## Finalization

- Draft creation by system engineers

- Review by software, safety, QA

- Comments resolved

- Baseline under CM, version v1.0 approved

## Traceability

| SRD | HLR |
| --- | --- |
| SYS-REQ-01 | HLR-01 |
| SYS-REQ-02 | HLR-02 |

# 2. High-Level Requirements (HLR)

## Purpose

- Define software-level requirements derived from SRD.

- Translate system requirements into software-specific, testable statements.

## Inputs

- SRD, ICD, ConOps, safety assessment, performance and timing specs

## Example Requirements

- HLR-01: Software shall sample temperature sensor every 1 second.

- HLR-02: Software shall detect over-temperature when temp > 80°C.

- HLR-03: Software shall clear over-temperature warning when temp < 75°C.

- HLR-04: Software shall set an over-temperature warning flag as Boolean.

## Finalization

- Peer review

- Fix comments

- Baseline in CM, approved

## Traceability

| SRD | HLR |
| --- | --- |
| SYS-REQ-01 | HLR-01 |
| SYS-REQ-02 | HLR-02 |

# 3. Low-Level Requirements (LLR)

## Purpose

- Define detailed logic to implement HLR.

- Basis for software design and coding.

## Inputs

- HLR, safety rules, timing constraints, ICD, environmental conditions

## Example Requirements

- LLR-01: Sample temperature sensor every 1000 ms.

- LLR-02: If temperature > 80°C for 2 consecutive samples, set flag TRUE.

- LLR-03: If temperature ≤ 75°C for 2 consecutive samples, set flag FALSE.

- LLR-04: Warning flag stored as Boolean.

## Finalization

- Technical review

- Fix defects

- Baseline under CM

## Traceability

| HLR | LLR |
| --- | --- |
| HLR-02 | LLR-02 |
| HLR-03 | LLR-03 |

# 4. Software Design Description (SDD)

## Purpose

- Describe architecture, modules, state machines, data flow.

- Define how LLR will be implemented in code.

## Inputs

- LLR, coding standards, hardware architecture, performance/timing, environmental constraints, ICD

## Example Design

- Module: TempMonitor

- States: NORMAL, OVER_TEMP

- Logic: Temp >80°C for 2 cycles → OVER_TEMP; Temp ≤75°C → NORMAL

- Data flow: Sensor → TempMonitor → Warning Flag

## Finalization

- Design review

- Fix comments

- Baseline in CM

## Traceability

| LLR | Design Element |
| --- | --- |
| LLR-02 | TempMonitor State Logic |
| LLR-03 | Reset Logic |

# 5. Software Verification Cases & Procedures (SVCP)

## Purpose

- Define test cases for verifying HLR/LLR.

- Include steps, inputs, expected results.

## Inputs

- HLR, LLR, SDD, coverage requirements

## Example Test Cases

- TC-01: Temp = 60°C → flag FALSE

- TC-02: Temp = 85°C → flag TRUE

- TC-03: Temp = 70°C → flag FALSE

- TC-04: Temp = 80°C → flag FALSE (boundary)

## Finalization

- Review by verification team + QA

- Ensure coverage of all requirements

- Baseline in CM

## Traceability

| LLR | Test Case |
| --- | --- |
| LLR-02 | TC-02 |
| LLR-03 | TC-03 |

# 6. Software Verification Results (SVR)

## Purpose

- Record results of test execution.

- Provide evidence that software satisfies requirements.

## Inputs

- SVCP, logs, coverage reports, anomaly reports

## Example Results

| Test Case | Result |
| --- | --- |
| TC-01 | PASS |
| TC-02 | PASS |
| TC-03 | PASS |
| TC-04 | PASS |

Coverage: Statement 100%, Branch 100%

## Finalization

- Review by verification lead + QA

- Fix open anomalies

- Baseline in CM

## Traceability

| Test Case | Result |
| --- | --- |
| TC-02 | PASS |

# 7. Software Validation Document (VDD / SVD)

## Purpose

- Summarize that software meets operational requirements.

- Confirm all requirements tested and passed.

- Provide compliance evidence for certification.

## Inputs

- HLR, LLR, SVCP, SVR, traceability, coverage, anomaly reports

## Content

- Validation strategy

- Summary of executed tests

- Coverage statistics

- Confirmation that all requirements validated

- No open anomalies

## Finalization

- Review by verification + QA

- Baseline under CM

- Approved for certification

## Traceability

| SRD | HLR | LLR | Design | Code | Test | Result |
| --- | --- | --- | --- | --- | --- | --- |
| SYS-REQ-02 | HLR-02 | LLR-02 | TempMonitor | TempMonitor() | TC-02 | PASS |
| SYS-REQ-03 | HLR-03 | LLR-03 | Reset Logic | TempMonitor() | TC-03 | PASS |

# Final Traceability Chain

SRD → HLR → LLR → SDD → Code → SVCP → SVR → Software Validation Document