MIL - STD -461

Conducted Emissions (CE)

CE101 – Low Frequency Conducted Emissions

Frequency: 30 Hz to 10 kHz

Measures noise on power leads

Focus: Harmonics from power supply, motor drives

CE102 – High Frequency Conducted Emissions

Frequency: 10 kHz to 10 MHz (sometimes up to 10–20 MHz depending setup)

Measures RF noise on power cables

Focus: Switching power supplies, digital circuits

🔵 Conducted Susceptibility (CS)

CS101 – Low Frequency Susceptibility

Frequency: 30 Hz to 50 kHz

Injects noise into power lines

Checks if system malfunctions due to ripple or low-frequency disturbance

CS114 – Bulk Cable Injection

Frequency: 10 kHz to 200 MHz

Injects RF current into cables using current injection probe

Tests immunity to RF interference on all signal/power cables

CS115 – Impulse Excitation

Applies fast transient pulses

Tests immunity to sudden spikes (like switching, relay noise)

CS116 – Damped Sinusoidal Transients

Frequency: 10 kHz to 100 MHz

Applies damped sine wave pulses

Simulates lightning-like or switching transients

🟢 Radiated Emissions (RE)

RE101 – Magnetic Field Emissions

Frequency: 30 Hz to 100 kHz

Measures low-frequency magnetic fields

Focus: Transformers, motors, power lines

RE102 – Radiated Electric Field Emissions

Frequency: 10 kHz to 18 GHz (depending version)

Measures RF radiation from equipment

Focus: Digital circuits, clocks, switching noise

🟣 Radiated Susceptibility (RS)

RS101 – Magnetic Field Susceptibility

Frequency: 30 Hz to 100 kHz

Tests immunity to low-frequency magnetic fields

RS103 – Radiated RF Susceptibility

Frequency: 2 MHz to 18 GHz

High-power RF field applied to system

Checks if system still works under RF exposure

📌 Quick Summary Table

| Category | Test | Purpose |
| --- | --- | --- |
| CE | CE101 | Low-frequency noise on power lines |
| CE | CE102 | High-frequency emissions on power lines |
| CS | CS101 | Low-frequency immunity |
| CS | CS114 | RF injection on cables |
| CS | CS115 | Pulse/transient immunity |
| CS | CS116 | Damped sine transients |
| RE | RE101 | Low-frequency magnetic radiation |
| RE | RE102 | RF radiation emissions |
| RS | RS101 | Magnetic immunity |
| RS | RS103 | RF field immunity |

Conducted Emissions (CE) – Noise coming OUT of your equipment

CE101 – Low Frequency Power Line Emissions

Frequency range: 30 Hz – 10 kHz

What it measures: Low-frequency noise/harmonics on power cables

Why it happens:

Transformers

Motor drives

DC-DC converters ripple

How test is done:

Current probe or LISN (Line Impedance Stabilization Network)

Measures current flowing back into power source

Impact: Can disturb other equipment sharing same power bus (e.g., aircraft DC bus)

✔ Key idea: “Are you polluting the power line at low frequency?”

CE102 – High Frequency Power Line Emissions

Frequency range: 10 kHz – 10 MHz (sometimes higher in setups)

What it measures: RF noise coming out of power lines

Sources:

Switching regulators

Digital clock harmonics

Fast edge signals

Test method:

LISN connected to power input

Spectrum analyzer measures RF noise

Impact: Can interfere with radios, communication systems

✔ Key idea: “Is your switching noise leaking into the power cable?”

🔵 Conducted Susceptibility (CS) – Noise coming INTO your equipment

CS101 – Low Frequency Power Line Immunity

Frequency: 30 Hz – 50 kHz

What it tests: Can system survive power ripple/noise?

Method:

Inject AC ripple into power lines

Observe system behavior

Typical issues:

Reset of microcontroller

ADC errors

Fixes:

Better power filtering

DC-DC regulator design improvement

✔ Key idea: “Will your system still work if power is noisy?”

CS114 – Bulk Cable Injection (RF Immunity)

Frequency: 10 kHz – 200 MHz

What it tests: RF noise entering through cables

Method:

Current injection probe clamps cable

Inject RF signal

Impact:

Data corruption

Communication errors (CAN, UART, etc.)

✔ Key idea: “Are your cables acting like antennas?”

CS115 – Impulse (Fast Pulse Immunity)

What it is: Very fast transient pulse injection

Purpose: Simulates switching spikes, relay arcs

Method:

Apply repetitive fast pulses on power/signal lines

Failure signs:

Reset

Glitches

Fix:

TVS diodes

RC snubbers

✔ Key idea: “Can your system survive sudden spikes?”

CS116 – Damped Sinusoidal Transients

Frequency: 10 kHz – 100 MHz

What it simulates:

Lightning indirect effects

Switching surges

Method:

Inject oscillating decaying waveform into cables

Impact:

Damage or malfunction in sensitive circuits

✔ Key idea: “Can your system survive oscillating surge noise?”

🟢 Radiated Emissions (RE) – Noise your device radiates OUT

RE101 – Low Frequency Magnetic Field Emissions

Frequency: 30 Hz – 100 kHz

What it measures: Magnetic field leakage

Sources:

Transformers

Inductors

Power loops

Test method:

Loop antenna placed near equipment

Impact:

Can affect nearby sensors (compasses, analog circuits)

✔ Key idea: “Are you leaking magnetic fields?”

RE102 – Radiated RF Emissions

Frequency: 10 kHz – GHz range

What it measures: RF radiation into air

Sources:

Clock signals

High-speed digital lines

Switching circuits

Test method:

Anechoic chamber

Antennas measure radiated energy

Impact:

Interference with radios, GPS, communication systems

✔ Key idea: “Is your board acting like a radio transmitter?”

🟣 Radiated Susceptibility (RS) – Noise coming INTO system through air

RS101 – Magnetic Field Immunity

Frequency: 30 Hz – 100 kHz

What it tests: Resistance to low-frequency magnetic fields

Method:

Large loop antenna generates magnetic field

Failure:

Sensor errors

Display flicker

✔ Key idea: “Can external magnetic fields disturb your system?”

RS103 – RF Radiated Immunity

Frequency: 2 MHz – 18 GHz

What it tests: Strong RF fields hitting equipment

Method:

Anechoic chamber

High-power antennas

Impact:

Communication failure

CPU reset

Data corruption

✔ Key idea: “Will your system survive in a strong RF environment?”

📌 Overall Picture (Very Important for Interviews)

CE / RE → emissions (what YOU produce)

CS / RS → susceptibility (what YOU tolerate)

| Category | Meaning |
| --- | --- |
| CE | Conducted emissions on cables |
| RE | Radiated emissions into air |
| CS | Conducted immunity (cables) |
| RS | Radiated immunity (air) |

1. What is MIL-STD-461C?

Answer:
It is a military standard for EMI/EMC control that defines limits and test methods for electronic equipment to ensure it does not emit excessive electromagnetic noise and is immune to external interference.

It covers:

Conducted emissions (CE)

Radiated emissions (RE)

Conducted susceptibility (CS)

Radiated susceptibility (RS)

2. Why is EMI/EMC testing required?

Answer:
To ensure:

Equipment does not interfere with other systems

System works reliably in noisy electromagnetic environments

Safety in critical systems like avionics, defense, and automotive

3. What is the difference between emissions and susceptibility?

Answer:

Emissions (CE/RE): Noise generated by the device

Susceptibility (CS/RS): Device’s ability to tolerate external noise

✔ Simple line:
Emissions = what you send out, Susceptibility = what you can handle

4. What is CE101 and CE102?

Answer:

CE101: Low-frequency conducted emissions on power lines (30 Hz–10 kHz)

CE102: High-frequency conducted emissions on power lines (10 kHz–10 MHz)

They measure noise traveling through power cables.

5. What is RE102?

Answer:
It measures radiated RF emissions from equipment over a wide frequency range (typically kHz to GHz). It ensures the device does not act like a transmitter.

🔵 Intermediate Level Questions

6. What is CS114 used for?

Answer:
CS114 is bulk cable injection testing used to check immunity against RF interference coupled into cables.

Frequency: 10 kHz to 200 MHz

Method: RF current injected using clamp probe

Purpose: Simulates real-world RF coupling into wiring

7. Why is shielding important in EMC design?

Answer:
Shielding:

Blocks external RF signals from entering system

Prevents internal noise from radiating out

Improves immunity and reduces emissions

Materials used:

Metal enclosures

Shielded cables

PCB ground planes

8. What is the purpose of LISN?

Answer:
A Line Impedance Stabilization Network (LISN):

Provides a fixed impedance for testing

Isolates DUT from external noise

Measures conducted emissions on power lines

9. What causes EMI in embedded systems?

Answer:
Common sources:

Switching power supplies (SMPS)

High-speed clocks (MCU/FPGA)

Fast switching MOSFETs

Long PCB traces acting as antennas

10. What is CS115?

Answer:
CS115 tests immunity against fast transient pulses applied to cables.

Simulates switching spikes and noise

Checks system stability under sudden disturbances

🟣 Advanced Level Questions

11. What is the difference between CE102 and RE102?

Answer:

| CE102 | RE102 |
| --- | --- |
| Conducted emissions on power lines | Radiated emissions into air |
| Measured using LISN | Measured using antenna in chamber |
| Noise travels via cables | Noise travels via electromagnetic waves |

12. How do PCB design choices affect EMI?

Answer:
Poor design increases EMI due to:

Large current loops → radiated noise

Improper grounding → noise coupling

Long traces → antenna effect

Good design practices:

Solid ground plane

Short return paths

Proper decoupling capacitors

Controlled impedance routing

13. What happens if a product fails RS103?

Answer:
RS103 failure means system is not immune to strong RF fields.

Symptoms:

System resets

Communication errors

Display flickering

Fixes:

Shielding enclosure

Better filtering on I/O lines

Improved grounding

14. How do you reduce conducted emissions?

Answer:

Add LC filters at power input

Use ferrite beads

Improve PCB layout (reduce loop area)

Slow down switching edges if possible

Proper grounding

15. What is the purpose of RS101?

Answer:
RS101 tests immunity against low-frequency magnetic fields (30 Hz–100 kHz). It ensures system is not affected by transformers or power-line magnetic fields.

16. Why does switching power supply create EMI?

Answer:
A switching power supply (SMPS) generates EMI because of:

High dv/dt and di/dt switching edges

Rapid switching of MOSFETs

Harmonics of switching frequency

These fast transitions create:

Conducted noise on power lines (CE102)

Radiated emissions from PCB traces (RE102)

✔ Key idea: Faster edges = more EMI

17. What is a ground loop and why is it bad?

Answer:
A ground loop happens when:

Multiple ground paths exist between two points

Current flows in unintended loop paths

Problems:

Acts like antenna → radiates EMI

Causes noise coupling into signals

Affects ADC accuracy and communication

✔ Fix:

Single-point grounding (star ground)

Ground plane design in PCB

18. What is common-mode and differential-mode noise?

Answer:

🔹 Differential-mode noise:

Noise between two signal lines

Example: CAN_H vs CAN_L noise

🔹 Common-mode noise:

Same noise appears on both lines relative to ground

Often caused by external RF interference

✔ Important:

Filters must target both modes differently

19. Why is decoupling capacitor important in EMI control?

Answer:
Decoupling capacitors:

Provide local energy storage

Reduce high-frequency noise on power lines

Short high-frequency current to ground

✔ Placement rule:

Must be placed very close to IC power pins

20. What happens if PCB trace length increases?

Answer:
Long traces behave like antennas:

Increase radiated emissions (RE102 failure)

Increase susceptibility to external noise (RS103 failure)

Add inductance → signal distortion

✔ Rule:

Shorter traces = lower EMI

⚡ Real Troubleshooting Questions

21. Your product fails CE102. How do you debug it?

Answer:
Step-by-step approach:

Use spectrum analyzer + LISN to identify frequency spikes

Trace source:

SMPS switching frequency?

Clock harmonics?

Isolate sections (power vs digital)

Fixes:

Add LC filter at power input

Add ferrite bead

Reduce switching edge speed

Improve grounding

✔ Key idea: Find frequency → trace source → fix root cause

22. Your system fails RS103 test. What do you check?

Answer:
RS103 failure means RF is affecting system.

Steps:

Identify sensitive subsystem (MCU, ADC, comm lines)

Check coupling path:

Cable acting as antenna?

PCB trace too long?

Apply fixes:

Shielding enclosure

Twisted pair cables

RC filtering on inputs

Ferrite beads on I/O

23. How does PCB stack-up affect EMI?

Answer:
PCB stack-up is critical:

Ground plane reduces loop area → lowers EMI

Power plane provides low impedance return path

Improper layer arrangement increases coupling

✔ Best practice:

Signal layer adjacent to ground plane

Avoid split ground under high-speed signals

24. Why do high-speed signals create EMI?

Answer:
Not because of frequency, but because of:

Fast edge rate (rise/fall time)

Harmonics extend into RF range

✔ Example:
A 10 MHz clock with fast edges can produce harmonics up to GHz range → RE102 failure

25. What is the role of shielding in EMI control?

Answer:
Shielding:

Blocks external electric fields (RS tests)

Prevents internal radiation (RE tests)

Types:

Cable shielding (braided/shielded cables)

Enclosure shielding (metal box)

PCB shielding (ground planes)

✔ Important: Shield must be properly grounded

⚡ Real Design Scenario Questions

26. How do you design a PCB to pass MIL-STD-461C?

Answer:
Key design steps:

Minimize loop areas

Use solid ground plane

Add EMI filters at power entry

Separate analog and digital sections

Use decoupling capacitors near ICs

Control clock routing

Use shielded connectors

27. Why does cable length matter in EMI?

Answer:
Long cables:

Act as antennas (radiate and receive noise)

Increase susceptibility (RS114 failure)

Increase coupling area

✔ Solution:

Shielded cables

Ferrite clamps

Proper grounding at one end

28. What is the most common reason for EMI failure?

Answer:
Top reasons:

Poor PCB layout (80% cases)

Missing or wrong grounding

Improper filtering

Long signal loops

Fast switching edges

29. How do you reduce EMI in digital systems?

Answer:

Slow down edge rates (if possible)

Add series resistors

Use ground planes

Filter power lines

Proper decoupling

Shield critical sections

30. Why is EMI harder in modern electronics?

Answer:
Because:

Higher switching speed

Higher density PCB

Lower voltage margins

More wireless coexistence

✔ Result: More harmonics + more coupling issues

31. What is the difference between near-field and far-field EMI?

Answer:

🔹 Near-field (close to source):

Distance: very close to PCB (few cm)

Dominated by electric or magnetic fields separately

Hard to predict radiation pattern

🔹 Far-field (far from source):

Distance: > wavelength/2π

Behaves like electromagnetic wave

Measured in RE102 tests

✔ Key idea:

Near-field = design problem

Far-field = radiation problem

32. Why does ground plane reduce EMI?

Answer:
A continuous ground plane:

Provides low impedance return path

Reduces loop area of current

Prevents radiation of high-frequency currents

✔ Without ground plane:

Current loops become large → acts like antenna → EMI increases

33. What is return path and why is it important?

Answer:
Every signal current must return to source.

If return path is:

Short + under trace → good (low EMI)

Long or broken → loop increases → radiation increases

✔ Key concept:

Signal + return path together form a loop antenna

34. Why do split ground planes cause EMI issues?

Answer:
If signal crosses split:

Return current must detour

Loop area increases

Radiated emissions increase (RE102 failure risk)

✔ Best practice:
Avoid splitting ground under high-speed signals.

35. What is ferrite bead and where is it used?

Answer:
A ferrite bead:

Acts as frequency-dependent resistor

Low impedance at DC

High impedance at RF

✔ Used for:

Power input filtering

Signal line noise suppression

⚡ Debugging & Failure Analysis Questions

36. How do you debug RE102 failure?

Answer:
Step-by-step:

Identify frequency peak using spectrum analyzer

Match frequency with:

Clock signal?

Switching supply?

Locate physical source on PCB

Apply fixes:

Shielding

Reduce edge speed

Add series resistor

Improve grounding

✔ Key idea: Frequency tells the source

37. If system passes CE but fails RE, what does it mean?

Answer:

CE pass → power line is clean

RE fail → radiation through air

✔ Interpretation:
Problem is internal radiation, not power supply

✔ Likely causes:

Clock lines

High-speed buses

PCB loop radiation

38. If system fails CS114, what is the issue?

Answer:
CS114 failure means:

External RF is coupling into cables

System is not immune to RF injection

✔ Causes:

Poor cable shielding

No common-mode filtering

Long unshielded wires

39. Why does system reset under RS103 test?

Answer:
Because RF field:

Couples into reset line or clock

Disturbs MCU operation

Causes voltage glitches

✔ Fix:

RC filter on reset line

Shielding

Improve decoupling

40. What is EMI coupling mechanism?

Answer:
There are 4 types:

Conductive coupling → through wires

Capacitive coupling → electric field

Inductive coupling → magnetic field

Radiative coupling → electromagnetic waves

✔ Most PCB problems = capacitive + inductive coupling

⚡ Design-Oriented Questions

41. How does switching frequency affect EMI?

Answer:

Higher frequency → more harmonic content

Faster edges → more high-frequency noise

EMI increases if not filtered properly

✔ Trade-off:

Efficiency vs EMI

42. Why is star grounding used?

Answer:
Star grounding:

All grounds meet at single point

Prevents ground loop currents

Reduces noise coupling

✔ Used in:

Analog systems

Mixed-signal PCB designs

43. What is snubber circuit and why is it used?

Answer:
A snubber (RC/RCD circuit):

Reduces voltage spikes

Controls switching transients

Reduces EMI from inductive loads

✔ Used across:

MOSFETs

Relays

Transformers

44. Why do high-speed clocks cause EMI?

Answer:
Not just frequency, but:

Fast rise/fall time → high harmonics

Harmonics extend into RF band → RE102 failure

✔ Fix:

Spread spectrum clocking

Series damping resistor

45. What is common-mode choke?

Answer:
A component that:

Blocks common-mode noise

Allows differential signal to pass

✔ Used in:

USB

Ethernet

Power lines

⚡ Real Interview Scenario Questions

46. Your system passes lab test but fails in field. Why?

Answer:
Possible reasons:

Real environment has stronger RF sources

Cable routing differs

Grounding conditions different

Temperature affects noise behavior

✔ Conclusion:
Lab ≠ real-world EMI environment

47. How do you reduce EMI at PCB level without redesign?

Answer:
Quick fixes:

Add ferrite beads

Add decoupling capacitors

Add shielding tape/enclosure

Add series resistors on fast signals

Improve grounding jumpers

48. Why is EMI worse at higher data rates?

Answer:
Because:

Faster transitions → higher frequency harmonics

More spectral content → wider EMI bandwidth

49. What is EMI filter made of?

Answer:
Typical EMI filter:

Inductor (blocks high frequency)

Capacitor (shunts noise to ground)

Sometimes resistor for damping

50. What is most important factor in EMI design?

Answer:
✔ Current loop area

Smaller loop:

Less radiation

Better immunity

Lower EMI