Communication Protocols Overview with Detailed History

1. CAN (Controller Area Network)

History

- Developed by Bosch in 1983 for automotive applications to allow microcontrollers and devices to communicate without a host computer.

- Standardized as ISO 11898 in 1993.

- Initially used in vehicles for engine control, ABS, and transmission systems.

- Expanded to industrial automation, medical devices, and robotics due to its robustness.

Key Features

- Multi-master serial bus

- Priority-based message arbitration

- Error detection and handling

- Real-time deterministic communication

Architecture

- Physical layer: twisted pair wires (CAN_H, CAN_L)

- Data link layer: CAN protocol with frames (standard 11-bit, extended 29-bit IDs)

- Supports up to 1 Mbps in classical CAN

2. RS-485

History

- Introduced in 1983 as an ANSI/TIA/EIA standard.

- Designed for long-distance, multi-drop serial communication in industrial environments.

- Popular in factory automation, process control, and building automation.

- Often used as the physical layer for protocols like Modbus RTU.

Key Features

- Differential signaling for noise immunity

- Multi-point (up to 32 devices)

- Half-duplex or full-duplex

- Baud rates up to 10 Mbps (short distance)

Architecture

- Physical layer: twisted pair cables

- Supports point-to-multipoint topology

- Protocols like Modbus RTU often run over RS-485

3. MQTT (Message Queuing Telemetry Transport)

History

- Created by IBM in 1999 for connecting sensors over networks with low bandwidth and high latency.

- Originally designed for oil pipelines monitoring.

- Became popular for IoT due to its lightweight publish/subscribe model.

- Standardized as ISO/IEC 20922 in 2016.

Key Features

- Publish/subscribe model

- Small footprint, low bandwidth usage

- QoS levels: 0, 1, 2

- Retained messages and last-will feature

Architecture

- Broker-based architecture

- Clients publish/subscribe to topics

- TCP/IP transport

4. TCP/IP (Transmission Control Protocol / Internet Protocol)

History

- Developed by DARPA(Defense Advanced Research Projects Agency) in the 1970s.

- Standardized in 1983 as the protocol suite for ARPANET.

- Foundation of modern internet communication.

- Widely adopted in industrial networks and embedded devices.

Key Features

- Reliable, connection-oriented communication (TCP)

- Unreliable, connectionless communication (UDP)

- IP addressing, routing, and fragmentation

- End-to-end data integrity

Architecture

- Layered: Application, Transport, Internet, Link

- Supports Ethernet, Wi-Fi, cellular, etc.

5. Modbus RTU

History

- Developed by Modicon (Schneider Electric) in 1979 for programmable logic controllers (PLCs).

- Designed as a simple serial protocol for industrial automation.

- Quickly became widely supported across multiple devices and vendors.

- Evolved into Modbus TCP for Ethernet networks.

Key Features

- Master/slave architecture

- Serial communication (RS-232, RS-485)

- Simple, widely supported

- Deterministic polling cycle

Architecture

- Data encoded in binary format (RTU)

- CRC for error checking

- Function codes for read/write operations

6. OPC UA (Open Platform Communications Unified Architecture)

History

- Developed by OPC Foundation in 2008.

- Successor to classic OPC, designed to overcome limitations of COM/DCOM.

- Platform-independent and secure industrial communication.

- Gained adoption in Industry 4.0 for device interoperability.

Key Features

- Client/server and pub/sub models

- Platform-independent, language-agnostic

- Security: encryption, authentication, integrity

- Scalable: embedded to enterprise systems

Architecture

- Service-oriented architecture

- Information modeling for devices and processes

- Transport via TCP, HTTPS, or WebSockets

7. BACnet (Building Automation and Control Networks)

History

- Developed in 1987 by ASHRAE(American Society of Heating, Refrigerating and Air-Conditioning Engineers) for HVAC, lighting, and building automation systems.

- Standardized as ANSI/ASHRAE 135.

- Designed to enable multi-vendor interoperability.

- Expanded to support modern IP-based networks (BACnet/IP).

Key Features

- Multi-vendor interoperability

- Object-oriented data representation

- Support for multiple physical layers (Ethernet, MS/TP, BACnet/IP)

- Event-driven communication

Architecture

- Network layer, application layer

- Uses objects and properties model

- Supports client/server and peer-to-peer communication

8. ARINC-429

History

- Developed by ARINC in 1977 for commercial aircraft avionics.

- Standard for data buses in aircraft systems like navigation, autopilot, and flight control.

- Designed for deterministic and robust avionics communication.

Key Features

- Simple, robust, point-to-point serial bus

- 32-bit word format: label, data, parity

- Supports up to 100 kbps data rate

- Deterministic and error-checked transmission

Architecture

- Transmitter sends data words continuously

- Receiver filters by label

- Unidirectional communication

This document provides detailed history, key features, and architecture for common embedded communication protocols including CAN, RS-485, MQTT, TCP/IP, Modbus RTU, OPC UA, BACnet, and ARINC-429.