### What do you mean by a real-time system?

A real-time system is a time-bound system that has well-defined fixed time constraints. Processing must be done within the defined constraints or the system will fail.

What is RTOS?

RTOS stands for Real-Time Operating System. It specially designed to run the application with very precise timing and a high degree of reliability. An RTOS system must have response time predictability and it must be deterministic.

### How is RTOS different from other OS?

RTOS offers services that allow tasks to be performed within predictable timing constraints.

### What are the different interprocess communications?

Semaphore, mutex, message passing, shared a memory, socket connections.

### What is the kernel?

The kernel is a computer program that is the core of a computer’s operating system, with complete control over everything in the system. On most systems, it is one of the first programs loaded on start-up (after the bootloader). It handles the rest of the start-up as well as input/output requests from software, translating them into data-processing instructions for the central processing unit. It handles memory and peripherals like keyboards, monitors, printers, and speakers.

What is multitasking in the OS?

Multitasking allows the user to run several tasks at the same time. However, only one task is active at a time for user interaction, but the processing ability of the computer’s processors is so fast and smooth that it gives the impression of performing multiple tasks at the same time.

What is the stack overflow?

If your program tries to access beyond the limit of the available stack memory then stack overflow occurs. In other words, you can say that a stack overflow occurs if the call stack pointer exceeds the stack boundary.

If stack overflow occurs, the program can crash or you can say that segmentation fault that is the result of the stack overflow.

### What is the cause of the stack overflow?

In the embedded application we have a little amount of stack memory as compare to the desktop application. So we have to work on embedded applications very carefully either we can face the stack overflow issues that can be a cause of the application crash.

Here, I have mentioned some causes of unwanted use of the stack.

1. Improper use of the recursive function.
2. Passing to many arguments in the function.
3. Passing a structure directly into a function.
4. Nested function calls.
5. Creating a huge size local array.

What is the core dump?

In computing, a core dump consists of the recorded state of the working memory of a computer program at a specific time, generally when the program has crashed or otherwise terminated abnormally.

On many operating systems, a fatal exception in a program automatically triggers a core dump. By extension, the phrase “to dump core” has come to mean in many cases, any fatal error, regardless of whether a record of the program memory exists. The term “core dump”, “memory dump”, or just “dump” has also become jargon to indicate any storing of a large amount of raw data for further examination or other purposes.

What is the volatile keyword?

The volatile keyword is a type qualifier that prevents the objects from the compiler optimization. According to the C standard, an object that has a volatile-qualified type may be modified in ways unknown to the implementation or have other unknown side effects. You can also say that the value of the volatile-qualified object can be changed at any time without any action being taken by the code. If an object is qualified by the volatile qualifier, the compiler reloads the value from memory each time it is accessed by the program that means it prevents from cache a variable into a register. Reading the value from memory is the only way to check the unpredictable change of the value.

What is the use of volatile keyword?

The volatile keyword is mainly used where we directly deal with GPIO, interrupt or flag Register. It is also used where a global variable or buffer is shared between the threads.

The Proper place to use the volatile keyword?

Here I am pointing some important places where we need to use the volatile keyword.

1. Accessing the memory-mapped peripherals register or hardware status register.

#define COM_STATUS_BIT 0x00000006

uint32_t const volatile * const pStatusReg = (uint32_t*)0x00020000;

unit32_t GetRecvData()

{

//Code to recv data

while (((*pStatusReg) & COM_STATUS_BIT) == 0)

{

// Wait until flag does not set

}

return RecvData;

}

2. Sharing the global variables or buffers between the multiple threads.

3. Accessing the global variables in an interrupt routine or signal handler.

volatile int giFlag = 0;

ISR(void)

{

giFlag = 1;

}

int main(void)

{

while (!giFlag)

{

//do some work

}

return 0;

}

What is ISR?

An ISR refers to the Interrupt Service Routines. These are procedures stored at specific memory addresses which are called when a certain type of interrupt occurs. The Cortex-M processors family has the NVIC (Nested Vectored Interrupt Controller) that manages the execution of the interrupt.

What is NVIC?

The NVIC (Nested Vectored Interrupt Controller) is an interrupt controller built inside ARM Cortex-M microcontrollers.
It manages hardware interrupts and exceptions efficiently.

It allows nested interrupts, meaning a higher-priority interrupt can preempt (interrupt) a lower-priority one

What are Nested Interrupts?

A nested interrupt occurs when an interrupt is being serviced, and another higher-priority interrupt arrives.

The CPU pauses the current ISR (Interrupt Service Routine), services the higher-priority ISR, and then resumes the lower-priority ISR from where it left off.

This ensures time-critical events are handled first.

How it Works (Step by Step)

Interrupt A (low priority) occurs → CPU jumps to ISR_A.

While executing ISR_A, a higher-priority Interrupt B arrives.

The NVIC compares priorities:

If B > A → CPU pushes context of ISR_A onto the stack.

CPU jumps to ISR_B.

After ISR_B finishes → CPU pops context and resumes ISR_A.

What is the endianness?

The endianness is the order of bytes to store data in memory and it also describes the order of byte transmission over a digital link. In the memory data store in which order depends on the endianness of the system, if the system is big-endian then the MSB byte store first (means at lower address) and if the system is little-endian then LSB byte store first (means at lower address).

Some examples of the little-endian and big-endian system.

### What is big-endian and little-endian?

Suppose, 32 bits Data is 0x11223344.

#### Big-endian

The most significant byte of data stored at the lowest memory address.

#### Little-endian

The least significant byte of data stored at the lowest memory address.

Note: Some processors can switch one endianness to other endianness using the software means it can perform like both big-endian or little-endian at a time. This processor is known as the Bi-endian, here are some architecture (ARM version 3 and above, Alpha, SPARC) who provide the switchable endianness feature.

Write a c program to check the endianness of the system?

#include <stdio.h>

#include <inttypes.h>

int main(void)

{

uint32_t data;

uint8_t *cptr;

data = 1; //Assign data

cptr = (uint8_t *)&data; //Type cast

if (*cptr == 1)

{

printf("little-endiann");

}

else if (*cptr == 0)

{

printf("big-endiann");

}

return 0;

}

Output:

Code Explanation:

If your machine is little-endian, the data in the memory will be something like the below expression:

higher memory

----->

+----+----+----+----+

|0x01|0x00|0x00|0x00|

+----+----+----+----+

^

|

&data

(uint8_t *)&data = 1;

But if your machine is big-endian, it will look like the below expression:

+----+----+----+----+

|0x00|0x00|0x00|0x01|

+----+----+----+----+

^

|

&data

(uint8_t *)&data = 0;

### How to Convert little-endian to big-endian vice versa in C?

#include <stdio.h>

#include <inttypes.h>

//Function to change one endian to another

uint32_t ChangeEndianness(uint32_t u32Value)

{

uint32_t u32Result = 0;

u32Result |= (u32Value & 0x000000FF) << 24;

u32Result |= (u32Value & 0x0000FF00) << 8;

u32Result |= (u32Value & 0x00FF0000) >> 8;

u32Result |= (u32Value & 0xFF000000) >> 24;

return u32Result;

}

int main()

{

uint32_t u32CheckData = 0x11223344;

uint32_t u32ResultData =0;

//swap the data

u32ResultData = ChangeEndianness(u32CheckData);

//converted data

printf("0x%x\n",u32ResultData);

return 0;

}

Output:

0x44332211

### What is virtual memory?

Virtual memory is part of memory management techniques and it creates an illusion that the system has a sufficient amount of memory. In other words, you can say that virtual memory is a layer of indirection.

What is Thread?

A thread is a basic unit to which the operating system allocates processor time. A thread can execute any part of the process code, including parts currently being executed by another thread.

### What are the processes?

An application consists of one or more processes. A process, in the simplest terms, is an executing program. One or more threads run in the context of the process.

What are the differences between process and thread?

Threads differ from traditional multitasking operating-system processes in several ways:

- The processes are typically independent, while threads exist as subsets of a process.

- The processes carry considerably more state information than threads, whereas multiple threads within a process share process state as well as memory and other resources.

- The processes have separate address spaces, whereas threads share their address space.

- The processes interact only through system-provided inter-process communication mechanisms.

- Context switching between threads in the same process typically occurs faster than context switching between processes.

What is a reentrant function?

In computing, a computer program or subroutine is called reentrant if it can be interrupted in the middle of its execution and then safely be called again (“re-entered”) before its previous invocations complete execution. The interruption could be caused by an internal action such as a jump or call, or by an external action such as an interrupt or signal. Once the reentered invocation completes, the previous invocations will resume correct execution.

What is semaphore?

Semaphore is simply a variable that is non-negative and shared between threads. This variable is used to solve the critical section problem and to achieve process synchronization in the multiprocessing environment. A semaphore is a signaling mechanism, and a thread that is waiting on a semaphore can be signaled by another thread.

What is mutex?

A Mutex is a mutually exclusive object which protects the shared resources from simultaneous access by multiple threads or processes. It acts as a gatekeeper to a section of code allowing one thread in and blocking access to all others. This ensures that the code being controlled will only be hit by a single thread at a time.

Mutex work on the locking mechanism, the thread which locks the mutex can only unlock it. So you must release the mutex after its use either it causes serious issues.

### What will happen if a non-recursive mutex is locked more than once?

If a thread that had already locked a mutex, tries to lock the mutex again, it will enter into the waiting list of that mutex, which results in a deadlock. It is because no other thread can unlock the mutex.

Are binary semaphore and mutex same?

No both are different.

Binary Semaphore

A signaling mechanism (used for synchronization).

Can take values 0 or 1.

Used to signal between tasks or between ISR and task.

No ownership → any task can “give” (release) or “take” (acquire).

Often used for event notification (e.g., an ISR signals a task when data is ready).

Priority Inversion handling: Not supported.

✅ Example use:

ISR signals a task that new data has arrived on UART.

Task waits (takes semaphore) until ISR gives it.

Mutex (Mutual Exclusion Object)

A locking mechanism (used for resource protection).

Also binary in nature (locked/unlocked), but with ownership.

Only the task that locks (takes) the mutex can unlock (give) it.

Used to protect shared resources (e.g., shared memory, I²C bus, file).

Supports priority inheritance to avoid priority inversion.

✅ Example use:

Multiple tasks want to access the same SPI bus.

Mutex ensures only one task uses it at a time.

Priority Inversion (the problem)

Happens when a high-priority task is blocked by a lower-priority task that holds a shared resource (mutex).

If a medium-priority task keeps running in between, the high-priority task gets delayed even more.

Example:

Low-priority task (L) locks a mutex to use a shared SPI bus.

High-priority task (H) needs the same mutex → blocked, waiting for L to release it.

Meanwhile, a medium-priority task (M) (not using mutex) keeps running, since it has higher priority than L.

Result → H (the most important task) waits unnecessarily → priority inversion.

Priority Inheritance (the solution)

A technique to solve priority inversion.

When a low-priority task (L) holds a resource needed by a high-priority task (H), the RTOS temporarily boosts L’s priority to H’s level.

This ensures L finishes quickly and releases the mutex, allowing H to continue.

Once L releases the resource, its priority is restored back.

How to preserve stack space?

Here, I am mentioning a few points which save the stack. If you want to update this list then you can comment in the comment box.

- Don’t pass the structure variable in a function. Use the pointer or reference to passing it in a function.

- Instead of A() calling B() which calls C() which calls D(), have A() call B(), C(), and D() itself.

- If a function is only referenced once, mark it inline (assuming your compiler supports this).

- Turn on your compiler optimization.

- Increase your compiler optimization level.

How RTOS does the task switching from interrupts?

The ISR has to be implemented in a way that allows for a context switch. Typically, at the end of the ISR, there will be a call to an RTOS function that checks for and performs the context switch.

When the interrupt occurs, the CPU saves its context and jumps to the ISR. The way the context is saved varies among CPU families. When the ISR is complete, it should call a special RTOS routine that allows for a context switch after an ISR. If there is a higher priority task ready to run then this routine will perform a context switch. It will take the pre-interrupt context saved by the CPU and save it with TASK_1. Then it will get the saved context of TASK_2 and restore it into the CPU such that when the end-of-interrupt instruction is called, execution returns to the context of TASK_2.

Note: The details of all of this are very CPU and RTOS-dependent.

What is PendSV?

PendSV is an interrupt-driven request for system-level service. In an OS environment, use PendSV for context switching when no other exception is active.

What is SVCall?

A supervisor call (SVC) is an exception that is triggered by the SVC instruction. In an OS environment, applications can use SVC instructions to access OS kernel functions and device drivers.

What is SysTick?

A SysTick exception is an exception the system timer generates when it reaches zero. The software can also generate a SysTick exception. In an OS environment, the processor can use this exception as a system tick.

SysTick (System Timer) is a 24-bit countdown timer built into the Cortex-M core itself.

It is used to generate periodic interrupts — commonly used for:

RTOS tick generation (like in FreeRTOS or MQX),

Simple time delays, or

System timekeeping.

What is UsageFault (ARM)?

A UsageFault is an exception that occurs because of a fault related to instruction execution. This includes:

1. An undefined instruction
2. An illegal unaligned access
3. Invalid state on instruction execution
4. An error on exception return.

The following can cause a UsageFault when the core is configured to report them:

1. An unaligned address on word and halfword memory access
2. Division by zero.

Are Interrupts are left disabled during the entire initialization process?

Yes. According to the threadX RTOS, Interrupts must be disabled during the entire initialization process. If somehow interrupts enables, then unpredictable behavior may occur.

What Necessary Conditions Can Lead To A Deadlock Situation In A System?

Answer :

Deadlock situations occur when four conditions occur simultaneously in a system: Mutual exclusion; Hold and Wait; No preemption; and Circular wait.

What Is Multitasking?

Answer :

Multitasking is the process within an operating system that allows the user to run several applications at the same time. However, only one application is active at a time for user interaction, although some applications can run “behind the scene”.

What Are Some Pros And Cons Of A Command Line Interface?

Answer :

A command line interface allows the user to type in commands that can immediately provide results. Many seasoned computer users are well accustomed to using the command line because they find it quicker and simpler. The main problem with a command line interface is that users have to be familiar with the commands, including the switches and parameters that come with it. This is a downside for people who are not fond of memorizing commands.

What Is Caching?

Answer :

Caching is the processing of utilizing a region of fast memory for a limited data and process. A cache memory is usually much efficient because of its high access speed.

What Is An Assembler?

Answer :

An assembler acts as a translator for low level language. Assembly codes, written using mnemonic commands are translated by the Assembler into machine language.

What Are Interrupts?

Answer :

Interrupts are part of a hardware mechanism that sends a notification to the CPU when it wants to gain access to a particular resource. An interrupt handler receives this interrupt signal and “tells” the processor to take action based on the interrupt request.

What Is Gui?

Answer :

GUI is short for Graphical User Interface. It provides users with an interface wherein actions can be performed by interacting with icons and graphical symbols. People find it easier to interact with the computer when in a GUI especially when using the mouse. Instead of having to remember and type commands, users just click on buttons to perform a process.

What Is Preemptive Multitasking?

Answer :

Preemptive multitasking allows an operating system to switch between software programs. This in turn allows multiple programs to run without necessarily taking complete control over the processor and resulting in system crashes.

### What are the different states of a process?

A list of different states of process:

- New Process

- Running Process

- Waiting Process

- Ready Process

- Terminated Process

What is the difference between micro kernel and macro kernel?

Micro kernel: micro kernel is the kernel which runs minimal performance affecting services for operating system. In micro kernel operating system all other operations are performed by processor.

Macro Kernel: Macro Kernel is a combination of micro and monolithic kernel.

What is virtual memory?

Virtual memory is a very useful memory management technique which enables processes to execute outside of memory. This technique is especially used when an executing program cannot fit in the physical memory.

What is a thread?

A thread is a basic unit of CPU utilization. It consists of a thread ID, program counter, register set and a stack.

What is deadlock? Explain.

Deadlock is a specific situation or condition where two processes are waiting for each other to complete so that they can start. But this situation causes hang for both of them.

### Which are the necessary conditions to achieve a deadlock?

There are 4 necessary conditions to achieve a deadlock:

- Mutual Exclusion: At least one resource must be held in a non-sharable mode. If any other process requests this resource, then that process must wait for the resource to be released.

- Hold and Wait: A process must be simultaneously holding at least one resource and waiting for at least one resource that is currently being held by some other process.

- No preemption: Once a process is holding a resource ( i.e. once its request has been granted ), then that resource cannot be taken away from that process until the process voluntarily releases it.

- Circular Wait: A set of processes { P0, P1, P2, . . ., PN } must exist such that every P[ i ] is waiting for P[ ( i + 1 ) % ( N + 1 ) ].

### What are the advantages of multithreaded programming?

A list of advantages of multithreaded programming:

- Enhance the responsiveness to the users.

- Resource sharing within the process.

- Economical

- Completely utilize the multiprocessing architecture.

It makes the system more responsive and enables resource sharing. It leads to the use of multiprocess architecture.It is more economical and preferred.

What are the different scheduling algorithms 
First-Come, First-Served (FCFS) Scheduling - Non-preemptive
Shortest-Job-Next (SJN) Scheduling. -Can be preemptive or non-preemptive
Priority Scheduling. -Preemptive or Non-preemptive
Shortest Remaining Time. -Can be preemptive or non-preemptive
Round Robin(RR) Scheduling. -Preemptive
Multiple-Level Queues Scheduling.-Usually preemptive

What is deadlock?  
Deadlock is a situation when two or more processes wait for each other to finish and none of them ever finish.  Consider an example when two trains are coming toward each other on same track and there is only one track, none of the trains can move once they are in front of each other.  Similar situation occurs in operating systems when there are two or more processes hold some resources and wait for resources held by other(s).

Q:Difference between #include<> vs #include”filename:

The difference is in the location where the preprocessor searches for the included file.

For #include "filename" the preprocessor searches in the same directory as the file containing the directive. This method is normally used to include programmer-defined header files.

For #include <filename> the preprocessor searches in an implementation dependent manner, normally in search directories pre-designated by the compiler/IDE. This method is normally used to include standard library header files.

What is the difference between preemptive and non-preemptive scheduling?

Answer: Scheduling is divided into 2 categories; preemptive scheduling and non-preemptive scheduling.

| Preemptive Scheduling | Non-Preemptive Scheduling |
| --- | --- |
| Processes are allocated to the CPU for a limited period. | A process remains in the CPU till it gets entirely executed. |
| If there is a new process that comes with high priority, then the ongoing CPU process has to stop. | Here the priority of the process does not matter. The new process has to wait until the first process finishes execution. |
| There is a continuous switching between the running and ready states of the processes. | This is not the case in non-preemptive scheduling. |
| Preemptive schedules the process on the basis of their priority. | Non-preemptive uses the process burst time. |
| There is a high probability that the process with less priority will starve. | Here process with less burst time will starve. |

What is interrupt latency?*.

The term interrupt latency refers to the delay from the start of the interrupt request to the start of interrupt handler execution. In the Cortex-M3 processor, if the memory system has zero latency, and provided that the bus system design allows vector fetch and stacking to happen at the same time, the interrupt latency can be as low as 12 cycles

Microcontroller start up process?

1.Power-On Reset (POR) → MCU starts in reset state.

2.Reset Vector / Boot ROM → CPU checks a fixed address.

3.Bootloader (if enabled)

Stored in a reserved memory region (often in Flash or internal ROM).

Decides whether to:

Load/run user application (main()), or

Enter firmware update mode (UART, USB, CAN, etc.).

4.Startup Code (if bootloader jumps to application) → Initializes stack, clock, variables.

5.Main Program → Runs your application

- How many stacks does an RTOS typically have?

- An RTOS typically has:

- One stack per task (so number of stacks = number of tasks).

- Idle task stack (system stack).

- ISR/system stack (depends on CPU architecture)

- Q) Who actually runs the scheduler in an RTOS?

- The CPU runs the scheduler code, but it’s triggered by the tick interrupt or RTOS kernel events.

- Q) Where is the initial value stored for global/static variables?

- Global & static variables in C

- They live in data segments of program memory (not on the stack).

- Two main cases:

- Global/static with initial value (e.g., int x = 5;)

- The value 5 is stored in Flash/ROM (because it must survive resets).

- At startup, the C runtime (startup code) copies that value from Flash into RAM (data section).

- So at runtime, x lives in RAM.

- Global/static without initial value (e.g., int y; or static int y;)

- They go into the .bss section in RAM.

- The startup code initializes them to 0.

- Q) Why do we need both .bss and .data sections in memory layout?

- .data holds variables with non-zero initial values (copied from Flash to RAM).

- .bss holds variables with zero/no initial values (RAM cleared at startup).

- Q) Does an 8-bit microcontroller support structure padding?

- Yes — structure padding is handled by the compiler, not by the microcontroller itself.

- Even on an 8-bit MCU (like AVR, PIC, 8051, etc.), the compiler may insert padding inside structures to align data to word/byte boundaries that improve access efficiency.

- Q) What is the purpose of padding in structures or memory alignment?

- Padding ensures that variables are stored at memory addresses aligned to the CPU’s word size. This avoids misaligned access, improves performance, and prevents hardware faults, though it may waste some memory.

- Q) What typically triggers a context switch in an RTOS?

- A context switch is typically triggered by tick interrupts, task blocking/unblocking, task yield, or an ISR that makes a higher-priority task ready.

- Q) How does the RTOS differentiate between ISR and task context?

- The RTOS differentiates between ISR and task by which stack pointer is active and the interrupt nesting level. ISRs run in hardware/handler context, tasks run in software-managed task context.

- Q) Why is dynamic memory allocation discouraged in safety-critical systems?

- Dynamic memory allocation is discouraged because it is non-deterministic, can fragment memory, may fail at runtime, and makes the system less predictable, which is unacceptable for safety-critical applications.

- Q) How does the linker script influence stack and heap placement?

- The linker script defines where stack and heap are placed in RAM by setting their start addresses and sizes, ensuring they don’t overlap with other sections like .data or .bss.

- Q) What function runs before main() in an embedded system?

The startup code or reset handler runs before main() to initialize memory sections and perform low-level system setup

How is function itoa() written inC?

itoa(1567, str, 10) should return string "1567"

itoa(-1567, str, 10) should return string "-1567"

itoa(1567, str, 2) should return string "11000011111"

itoa(1567, str, 16) should return string "61f"

Anti-Aliasing Filters and Their Usage Explained

Ans: According to the Nyquist sampling theorem, the sampling rate should be at least twice the maximum frequency component of the signal of interest. In other words, the maximum frequency of the input signal should be less than or equal to half of the sampling rate.

How do you ensure that this is definitely the case in practice? Even if you are sure that the signal being measured has an upper limit on its frequency, pickup from stray signals (such as the powerline frequency or from local radio stations) could contain frequencies higher than the Nyquist frequency. These frequencies may then alias into the appropriate frequency range and thus give you erroneous results.

To be sure that the frequency content of the input signal is limited, a low pass filter (a filter that passes low frequencies but attenuates the high frequencies) is added before the sampler and the ADC. This filter is an anti-alias filter because by attenuating the higher frequencies (greater than the Nyquist frequency), it prevents the aliasing components from being sampled. Because at this stage (before the sampler and the ADC) you are still in the analog world, the anti-aliasing filter is an analog filter.

An ideal anti-alias filter passes all the appropriate input frequencies (below f1) and cuts off all the undesired frequencies (above f1). However, such a filter is not physically realizable. In practice, filters look as shown in illustration (b) below. They pass all frequencies < f1, and cut-off all frequencies > f2. The region between f1 and f2 is known as the transition band, which contains a gradual attenuation of the input frequencies. Although you want to pass only signals with frequencies < f1, those signals in the transition band could still cause aliasing. Therefore in practice, the sampling frequency should be greater than two times the highest frequency in the transition band. This turns out to be more than two times the maximum input frequency (f1). That is one reason why you may see that the sampling rate is more than twice the maximum input frequency

Responsibilities

- Design and implement software of embedded devices and systems from requirements to production and commercial deployment

- Design, develop, code, test and debug system software

- Review code and design

- Analyze and enhance efficiency, stability and scalability of system resources

- Integrate and validate new product designs

- Support software QA and optimize I/O performance

- Provide post production support

- Interface with hardware design and development

- Assess third party and open source software